Cyber Attacks in Intelligent Transportation System

I have read three research paper and list down the attacks mentioned in these papers:

Paper A - Data poisoning attacks in intelligent transportation systems
 Data-poisoning-attack

Paper B - Cyber Threats Facing Autonomous and Connected Vehicles
 Parkinson 2017

Paper C - Survey of attacks and detection mechanisms on VANETs and IoV

I have divided them into 5 groups depending upon the area of target of these cyber attacks:

GROUP 1 - AI / Data-Centric Attacks

GROUP 2 - Localization & Navigation Attacks

GROUP 3 - VANET / V2X Network Attacks

GROUP 4 - Intra-Vehicle / Internal Network Attacks

GROUP 5 - Large-Scale ITS Infrastructure Attacks

📌 GROUP 1 — AI / Data-Centric Attacks (ML-Based ITS Attacks)

| Attack Name | Target | Effect | Source Paper |
| --- | --- | --- | --- |
| Data Poisoning Attack (Training Stage) | ML training datasets | Model learns incorrect patterns → unsafe traffic prediction | Paper A |
| Online Data Poisoning (Decision-Time Attack) | Real-time decision systems | Wrong routing, signal timing errors | Paper A |
| False Data Injection (FDI) | Traffic sensors / state estimators | Incorrect congestion estimation | Paper A |
| Evasion (Adversarial Input) Attack | DL perception models | Traffic sign/object misclassification | Paper A |

📌 GROUP 2 — Localization & Navigation Attacks

| Attack Name | Target | Effect | Source Paper |
| --- | --- | --- | --- |
| GNSS Spoofing | GPS / Vehicle localization | Misleading vehicle location | Paper B |
| GNSS Jamming | GPS receiver | Loss of positioning service | Paper B |
| LiDAR Replay Attack | LiDAR sensor | Fake obstacle perception | Paper A |
| Camera Blinding Attack | Camera sensors | Loss of visual perception | Paper A |
| Sensor Fusion Poisoning | Multi-sensor localization systems | System-level navigation failure | Paper A |

📌 GROUP 3 — VANET / V2X Network Attacks

| Attack Name | Target | Effect | Source Paper |
| --- | --- | --- | --- |
| Sybil Attack | Vehicle identity system | Fake congestion / fake vehicles | Paper C |
| Node Impersonation Attack | Identity system | Pretending to be another vehicle | Paper C |
| Bogus Information Attack | V2V communication | False accident or traffic alerts | Paper C |
| Motorway Attacker | High-mobility VANET | Spreads false data across regions | Paper C |
| Blackhole Attack | Routing protocol | Drops packets intentionally | Paper C |
| Wormhole Attack | Routing topology | Fake shortest paths | Paper C |
| JellyFish Attack | Congestion control | Packet delay/reorder | Paper C |
| Intelligent Cheater Attack | Routing layer | Selective misbehavior | Paper C |
| Flooding Attack | Bandwidth / routing | Resource exhaustion | Paper C |
| Denial of Service (DoS) | RSU / network | Communication shutdown | Paper C |
| Distributed DoS (DDoS) | RSU / cloud | Large-scale disruption | Paper C |
| Wireless Jamming | VANET channel | Communication blockage | Paper C |

📌 GROUP 4 — Intra-Vehicle / Internal Network Attacks

| Attack Name | Target | Effect | Source Paper |
| --- | --- | --- | --- |
| CAN Bus Injection | Vehicle internal network | Falsified speed/control signals | Paper B |
| ECU Authentication Bypass | Electronic Control Units | Unauthorized commands | Paper B |
| Sensor Manipulation (IMU / Engine Sensors) | Control systems | Unsafe vehicle behavior | Paper B |
| TPMS Exploitation | Tire monitoring system | Tracking / spoofing | Paper B |

📌 GROUP 5 — Large-Scale ITS Infrastructure Attacks

| Attack Name | Target | Effect | Source Paper |
| --- | --- | --- | --- |
| Ghost Vehicle Attack | Network-level ITS | Artificial traffic congestion | Paper A |
| Traffic Prediction Poisoning | Traffic ML models | Reduced network efficiency | Paper A |
| V2X Data Manipulation | Vehicle communication | Platooning instability | Paper A |